Caledonian Cables

Automotive Cable

FL6Y2G

Application:

This FEP insulated multi-cores cable is used for automobiles.

Construction:

FEP insulation

Soft-annealed copper conductor

Silicone rubber,cross-linked sheath

Conductor:

Cu-ETP1 or bare according to DIN EN 13602

Insulation:

Fluorinated ethylene propylene(FEP) insulation

Sheath:

Siliconne rubber,cross-linked according to ISO 14572 class F

Standard:

ISO 6722 Class F

Technical Parameters:

Operating temperature: –40°C to +210°C

|  | Conductor |  |  | Insulation | Cable |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal cross- section | No. and Dia. of Wires | Diameter max. | Electrical resistance at 20°C max. | Thickness Wall nom. | Sheath Thickness | Overall Diameter min. | Overall Diameter max. | Weight approx. |
| mm2 | No./mm | (mm) | mΩ/m | (mm) | (mm) | mm | mm | kg/km |
| 2x0.35 | 12/0.21 | 0.8 | 52 | 0.4 | 0.53 | 4.6 | 5 | 32 |
| 2x0.25 | 24/0.16 | 0.7 | 86.5 | 0.4 | 0.53 | 3.4 | 3.8 | 24 |

Note: Other configurations, sizes, colors and length not specified herein are available upon request.

www.caledonian-cables.co.uk	www.addison-cables.com	27