Caledonian Cables

Automotive Cable

FLRYCY

Application:

This PVC insulated, PVC sheathed multi-cores cable is used for car communication cable.

Construction:

Drain wire PVC insulation

Copper conductor

Soft-annealed copper wires PVC sheath

Conductor: Cu-ETP1 according to DIN EN 13602

Insulation: PVC

Drain wire: Drain wire of tinned copper wires of Cu-ETP1

Shielding: Soft-annealed copper wires CU-ETP1 or tin coated soft-annealed copper wires according to DIN 40500 and DIN EN 13602

Sheath: PVC

Standard Compliance: ISO 6722

Technical Parameters:

Operating temperature: –40 °C to +105 °C

|  | Conductor |  |  | Insulation |  | Cable |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Cross- section | No. and Dia. of Wires | Diameter max. | Electrical Resistance at 20°C Bare/tinned max. | Thickness Wall nom. | Diameter of Core | Sheath Thickness | Overall Diameter min. | Overall Diameter max. | Weight approx. |
| mm2 | No./mm | mm | mΩ/m | mm | mm | mm | mm | mm | kg/km |
| 9 x0.08 | 10/0.11 | 0.45 | 35.30/36.50 | 0.20 | 0.80 | 0.60 | 4.60 | 4.90 | 38 |
| 10 x0.25 | 14/0.16 | 0.70 | 84.80/86.50 | 0.20 | 1.10 | 0.60 | 5.80 | 6.20 | 68 |
| 5 x0.35 | 19/0.16 | 0.80 | 52.00/54.50 | 0.25 | 1.30 | 0.50 | 4.70 | 5.10 | 47 |
| 8 x0.35 | 19/0.16 | 0.80 | 52.00/54.50 | 0.25 | 1.25 | 0.65 | 5.90 | 6.30 | 75 |
| 10 x0.35 | 19/0.16 | 0.80 | 52.00/54.50 | 0.25 | 1.25 | 0.65 | 6.50 | 6.90 | 83 |

Note: Other configurations, sizes, colors and length not specified herein are available upon request.

26	www.caledonian-cables.co.uk	www.addison-cables.com